PT.IndowijayaSaktiTeguh

Jl.PangeranJayakarta121/3-4,JakartaPusat www.puffinpaint.comTelp:021-62313238

# MATERIALSAFETY DATASHEET

PUFFINEPOXYANTISLIPCLEARBASEB

Published:	Januari2025	Product–identityno:IW-PEASB

Edited	:	-	Distributiondate	:January 2025

TRADENAME:PUFFINEPOXYANTISLIPCLEARBASEB

# IDENTIFICATIONOFSUBSTANCEANDCOMPANY

IdentificationofSubstanceorPreparation Chemical description: Polyamine Recommendation	: Hardener

Company/UndertakingIdentificaationSupplier

PT.IndowijayaSaktiTeguh

Jl.PangeranJayakartaNo.121/3-4,JakartaPusat,10730.Indonesia Phone	:	+6221 6231 3238

WA	:	+6287873743534(WAmessagesonly)

Email	:	sales.support@puffinpaint.com

# HAZARDIDENTIFICATION

Classificationofthesubstanceormixture

  - SKINCORROSION/IRRITATION-Category 2

  - SERIOUSEYEDAMAGE/EYEIRRITATION-Category2A

  - SKINSENSITISATION-Category1

  - LONG-TERM(CHRONIC)AQUATICHAZARD-Category2

GHSLabelelements Hazard pictograms:

Signal word : Warning Hazard statements:

Causesseriouseyeirritation. Causes skin irritation.

Maycauseanallergicskinreaction.

Toxictoaquaticlifewithlonglastingeffects

Precautionarystatements

Prevention: Wear protective gloves. Wear eye or face protection. Avoid release to the environment.Wash hands thoroughly after handling.

Response:Collect spillage. IF ON SKIN: Wash with plenty of soap and water.Take off contaminated clothing and wash it before reuse. If skin irritation or rash occurs: Get medical attention. IF IN EYES:Rinse cautiously with water for several minutes.Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists:Get medical attention.

P305 + P351 + P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Storage: Not applicable

Disposal: Dispose of contents and container in accordance with all local, regional, national and international regulations

Otherhazardswhichdonotresultinclassification:Noneknown

# COMPOSITION/INFORMATIONONINGREDIENTS:

Substance/mixture:mixture

Othermeansofidentification:Notavailable

CASnumber/otheridentifiers CAS number : Not applicable. EC number : Mixture.

Productcode:496

| Ingredientsname | % | Casnumber |
| --- | --- | --- |
| epoxyresin(MW≤ 700) oxirane,mono[(c12-14- alkyloxy)methyl]derivs Fatty acids, C18-unsatd., dimers, oligomericreactionproductswith1- chloro-2,3-epoxypropane Oxirane,2-(chloromethyl)-,polymerwith α-hydro-ωhydroxypoly[oxy(methyl-1,2- ethanediyl)] | ≥50-≤75 ≥10-≤25 ≤3 ≤3 | 1675-54-3 68609-97-2 68475-94-5 9072-62-2 |

Therearenoadditionalingredientspresentwhich,withinthecurrentknowledgeofthe supplier and in the concentrations applicable, are classified as hazardous to health or the environment and hence require reporting in this section.

Occupationalexposurelimits,ifavailable,arelistedinSection8. Chemical formula : Not applicable

# FIRSTAIDMEASURES

Descriptionoffirstaidmeasures

- Eyecontact:Immediatelyflusheyeswithplentyofwater,occasionallyliftingthe upper and lower eyelids. Check for and remove any contact lenses. Continue to rinse for at least 10 minutes. Get medical attention.

- Inhalation:Removevictimtofreshairandkeepatrestinapositioncomfortable for breathing. If not breathing, if breathing is irregular or if respiratory arrest occurs, provide artificial respiration or oxygen by trained personnel.It may be dangeroustothepersonprovidingaidtogivemouth-to-mouthresuscitation.Get medical attention if adverse health effects persist or are severe.If unconscious, place in recovery position and get medical attention immediately.Maintain an open airway.Loosen tight clothing such as a collar, tie, belt or waistband.In case of inhalation of decomposition products in a fire, symptoms may be delayed.The exposed person may need to be kept under medical surveillance for 48 hours.

- Skin contact : Wash with plenty of soap and water.Remove contaminated clothingandshoes.Washcontaminatedclothingthoroughlywithwaterbefore removing it, or wear gloves.Continue to rinse for at least 10 minutes.Get medical attention.In the event of any complaints or symptoms, avoid further exposure.Washclothingbeforereuse.Cleanshoesthoroughlybeforereuse.

- Ingestion:Washoutmouthwithwater.Removedenturesifany.Removevictim tofreshairandkeepatrestinapositioncomfortableforbreathing.Ifmaterialhas been swallowed and the exposed person is conscious, give small quantities of water to drink.Stop if the exposed person feels sick as vomiting may be dangerous.Do not induce vomiting unless directed to do so by medical personnel.Ifvomitingoccurs,theheadshouldbekeptlowsothatvomitdoesnot enter the lungs.Get medical attention if adverse health effects persist or are severe.Nevergiveanythingbymouthtoanunconsciousperson.Ifunconscious, place in recovery position and get medical attention immediately.Maintain an open airway.Loosen tight clothing such as a collar, tie, belt or waistband.

Mostimportantsymptomsandeffects,bothacuteanddelayed Potential acute health effects

Eyecontact:Causesseriouseyeirritation

Inhalation:Noknownsignificanteffectsorcriticalhazards.

Skincontact:Causesskinirritation.Maycauseanallergicskinreaction

Ingestion:Noknownsignificanteffectsorcriticalhazards.

Over-exposuresigns/symptoms

Eyecontact:Adversesymptomsmayincludethefollowing:painorirritationwatering redness

Inhalation:Nospecificdata.

Skincontact:Adversesymptomsmayincludethefollowing:irritationredness

Ingestion:Nospecificdata.

Indicationofimmediatemedicalattentionandspecialtreatmentneeded,if necessary

Notes to physician : In case of inhalation of decomposition products in a fire, symptomsmaybedelayed. Theexposedpersonmayneedtobekeptundermedical surveillance for 48 hours.

Specifictreatments:Nospecifictreatment.

Protection of first-aiders : No action shall be taken involving any personal risk or withoutsuitabletraining.Itmaybedangeroustothepersonprovidingaidtogivemouth- to-mouth resuscitation. Wash contaminated clothing thoroughly with water before removing it, or wear gloves.

# FIREFIGHTINGMEASURES

Extinguishingmedia

Suitable extinguishing media	:Useanextinguishingagentsuitableforthesurrounding fire.

Unsuitableextinguishingmedia:Noneknown

Specifichazardsarisingfromthechemical:Inafireorifheated,apressure increasewill occur and the container may burst. This material is toxic to aquatic life with long lasting effects.Fire water contaminated with this material must be contained and prevented from being discharged to any waterway, sewer or drain.

Hazardousthermaldecompositionproducts:Decompositionproductsmayincludethe following materials:

carbondioxide,carbonmonoxide,nitrogenoxides,sulfuroxides,halogenated compounds,carbonyl halides ,metal oxide/oxides

Hazardouscombustionproducts:Decompositionproductsmayincludethefollowing materials: carbon monoxide, carbon dioxide, smoke, oxides of nitrogen.

Special protective actions for fire-fighters : Promptly isolate the scene by removing all personsfromthevicinityoftheincidentifthereisa fire.Noactionshallbetakeninvolving any personal risk or without suitable training.

Special protective equipment for fire-fighters: Fire-fighters should wear appropriate protectiveequipmentandself-containedbreathingapparatus(SCBA)withafullface-piece operated in positive pressure mode.

# ACCIDENTALRELEASEMEASURES

Personal precautions, protective equipment and emergency procedures

For non-emergency personel: No actionshallbe takeninvolvinganypersonalrisk or without suitable training. Evacuate surrounding areas.Keep unnecessary and unprotected personnel from entering. Do not touch or walk through spilt material. Avoid breathing vapour or mist. Provide adequate ventilation.Wear appropriate respirator when ventilation is inadequate. Put on appropriate personal protective equipment.

For emergency responders : If specialised clothing is requiredtodealwith thespillage, take note of any information in Section 8 on suitable and unsuitable materials. See also the information in "For non-emergency personnel".

Environmental precautions: Avoiddispersalofspiltmaterialandrunoffandcontact with soil,waterways,drainsandsewers.Informtherelevantauthoritiesiftheproducthascaused environmental pollution (sewers, waterways, soil or air). Water polluting material.Maybe harmfultotheenvironmentifreleasedinlargequantities.Collectspillage.

Methodsandmaterialforcontainmentandcleaningup

Small spill : Stop leak if without risk. Move containers from spill area. Dilute with water and mop up if water-soluble. Alternatively, or if water-insoluble, absorb with an inert dry material and place in an appropriate waste disposal container.Dispose of via a licensed waste disposal contract

Large spill : Stop leak if without risk. Move containers from spill area. Approach the release from upwind. Prevent entry into sewers, water courses, basements or confined areas. Wash spillagesintoaneffluenttreatmentplantorproceedasfollows.Containandcollectspillagewith non-combustible, absorbent material e.g. sand, earth, vermiculite or diatomaceous earth and place in container for disposalaccording to local regulations (see Section13). Dispose ofvia a licensed waste disposal contractor. Contaminated absorbent material may pose the same hazard as the spilt product. Note: see Section 1 for emergency contact information andSection 13 for waste disposal.

# HANDLINGANDSTORAGE

Precautionsforsafehandling

Protectivemeasures:Putonappropriatepersonalprotectiveequipment(seeSection8). Personswithahistoryofskinsensitizationproblemsshouldnotbeemployedinanyprocessin whichthisproductisused.Donotgetineyesoronskinorclothing.Donotingest.Avoid breathing vapour or

mist.Avoidreleasetotheenvironment.Keepintheoriginalcontaineroranapproved alternativemadefromacompatiblematerial,kepttightlyclosedwhennotinuse.Empty containersretainproductresidueandcanbehazardous.Donotreusecontainer.

Adviceongeneraloccupationalhygiene:Eating,drinkingandsmokingshouldbeprohibited inareaswherethismaterialishandled,storedandprocessed.Workersshouldwashhands andfacebeforeeating,drinkingandsmoking.Removecontaminatedclothingandprotective equipmentbeforeenteringeatingareas.SeealsoSection8foradditionalinformationon hygiene measures.

Conditionsforsafestorage,includinganyincompatibilities:Storeinaccordancewithlocal regulations.Storeinoriginalcontainerprotectedfromdirectsunlightinadry,coolandwell- ventilatedarea,awayfromincompatiblematerials(seeSection10)andfoodanddrink.Keep container tightlyclosedandsealeduntilreadyforuse.Containersthathavebeenopenedmust becarefullyresealedandkeptuprighttopreventleakage.Donotstoreinunlabelledcontainers. Useappropriatecontainmenttoavoidenvironmentalcontamination.SeeSection10for incompatiblematerialsbeforehandlingoruse.

Emissionsfromventilationorworkprocessequipmentshouldbecheckedtoensurethey complywiththerequirementsofenvironmentalprotectionlegislation.Insomecases,fume scrubbers,filtersorengineeringmodificationstotheprocessequipmentwillbenecessaryto reduceemissionstoacceptablelevels

# EXPOSURECONTROLSPERSONALPROTECTION

Control parameters Occupationalexposurelimits None

Appropriateengineeringcontrols:Good general ventilation should be sufficient to control workerexposuretoairbornecontaminants

Environmental exposure controls : Emissionsfromventilationor workprocessequipment shouldbecheckedtoensuretheycomplywiththerequirementsofenvironmentalprotection legislation.Insomecases,fumescrubbers,filtersorengineeringmodificationstotheprocess equipmentwillbenecessarytoreduceemissionstoacceptablelevels.

Individualprotectionmeasures

Hygienemeasures:Washhands,forearmsandfacethoroughlyafterhandlingchemical products,beforeeating,smokingandusingthelavatoryandattheendoftheworkingperiod.

Appropriate

techniquesshouldbeusedtoremovepotentiallycontaminatedclothing.Contaminatedwork clothingshouldnotbeallowedoutoftheworkplace.Washcontaminatedclothingbefore reusing.Ensurethateyewashstationsandsafetyshowersareclosetotheworkstationlocation

Eye/faceprotection:SafetyeyewearcomplyingtoEN166shouldbeusedwhenarisk assessmentindicatesthisisnecessarytoavoidexposuretoliquidsplashes,mists,gasesor dusts.Ifcontactispossible,thefollowingprotectionshouldbeworn,unlesstheassessment indicatesahigherdegreeofprotection:chemicalsplashgoggles.

Skinprotection

Handprotection:Thereisnooneglovematerialorcombinationofmaterialsthatwillgive unlimitedresistancetoanyindividualorcombinationofchemicals.Thebreakthroughtimemust begreaterthantheendusetimeoftheproduct.Theinstructionsandinformationprovidedby theglovemanufactureronuse,storage,maintenanceandreplacementmustbefollowed.

Glovesshouldbereplacedregularlyandifthereisanysignofdamagetotheglovematerial. Alwaysensurethatglovesarefreefromdefectsandthattheyarestoredandusedcorrectly. Theperformanceoreffectivenessoftheglovemaybereducedbyphysical/chemicaldamage andpoormaintenance.Barriercreamsmayhelptoprotecttheexposedareasoftheskinbut shouldnotbeappliedonceexposurehasoccurred.WearsuitableglovestestedtoEN374.

Recommended,gloves(breakthroughtime)>8hours:PVC,fluorrubber,Viton®,PE,butyl rubber,nitrilerubber,neoprene

Forrightchoiceofglovematerials,withfocusonchemicalresistanceandtimeofpenetration, seekadvicebythesupplierofchemicalresistantgloves.Theusermustcheckthatthefinal choiceoftypeofgloveselectedforhandlingthisproductisthemostappropriateandtakesinto accounttheparticularconditionsofuse,asincludedintheuser'sriskassessment.

Bodyprotection:Personalprotectiveequipmentforthebodyshouldbeselectedbasedonthe taskbeingperformedandtherisksinvolvedandshouldbeapprovedbyaspecialistbefore handling this product.

Otherskinprotection:Appropriatefootwear andanyadditionalskinprotectionmeasures shouldbeselectedbasedonthetaskbeingperformedandtherisksinvolvedandshouldbe approvedbyaspecialistbeforehandlingthisproduct.

Respiratoryprotection:Basedonthehazardandpotentialforexposure,selectarespirator thatmeetstheappropriatestandardor certification.Respiratorsmustbeusedaccordingtoa respiratoryprotectionprogramtoensureproperfitting,training,andotherimportantaspectsof use.

# PHYSICALANDCHEMICALPROPERTIES

Appearaance

Physicalstate	:Liquid.

Colour	:Variouscolours

Odour	:Characteristic

Odourthreshold	: NotAvailable

PH	: NotApplicable

Melting/freezingpoint	:NotApplicable

Boilingpoint	:Lowestknownvalue:>260°C(>500°F)(epoxyresin(MW

≤700)).

Flash point	:Closedcup:100°C(212°F)

Burningtime	:Not applicable.

Burning rate	:Not applicable.

Evaporationrate	: Notavailable.

Flammability(solid,gas)	:Notapplicable

Lowerandupperexplosive(flammable)limits:Notapplicable

Vapour pressure	:Highestknownvalue:0kPa(0mmHg)(at20°C)(epoxy resin (MW ≤ 700)). Weighted average: 0 kPa (0 mm Hg) (at 20°C)

Vapour density	:Highestknownvalue:11.7(Air=1)(epoxyresin(MW≤ 700)).

Relativedensity	:1.17to1.392g/cm³

Solubility		:Insolubleinthefollowingmaterials:coldwaterandhot water.

Solubility in water	: Not available. Partitioncoefficient:noctanol/water:Notavailable. Auto-ignition temperature.		: Not available. Decomposition temperature	: Not available.

SADT	:Notavailable.

Viscosity	:Dynamic:Highestknownvalue:10to12cP(epoxyresin (MW ≤ 700))Weighted average:10.13 cP Kinematic (40C): >20.5 cSt

# STABILITYANDREACTIVITY

Reactivity:Nospecifictestdatarelatedtoreactivityavailableforthisproductoritsingredients.

ChemicalStability:Theproductisstable.

PossibilityofHazardousreaction:Undernormalconditionsofstorageanduse,hazardous reactions will not occur.

Conditionstoavoid:No specificdata.

Incompatiblematerials:Keepawayfromthefollowingmaterialstopreventstrongexothermic reactions: oxidising agents, strong alkalis, strong acids.

HazardousDecompositionProducts:Undernormalconditionsofstorageanduse,hazardous decomposition products should not be produced

SADT:Not available.

# TOXICOLOGICALINFORMATION

| Product/Ingredientname | Result | Species | Dose | Exposure |
| --- | --- | --- | --- | --- |
| epoxyresin(MW≤ 700) oxirane,mono[(c12-14-alkylox methyl] derivs | LD50Dermal LD50 Oral LD50 Oral | Rabbit Mouse Rat | 20g/kg 15600mg/kg 17100mg/kg | - - - |

Informationontoxicologicaleffects Acute toxicity

Irritation/corrosion

| Product/ingredientname | Result | species | score | exposure | observation |
| --- | --- | --- | --- | --- | --- |
| epoxyresin(MW≤700) oxirane,mono[(c12-14-alkyloxy) methyl] derivs Fattyacids,C18-unsatd.,dimers, oligomeric reaction products with1-chloro2,3-epoxypropane | Eyes-Severeirritant Skin - Mild irritant Skin-Moderateirritant Skin - Mild irritant Skin-Mildirritant | Rabbit Rabbit Rabbit Mammal species Mammal Species Unspecified | - - - - - | 24 hours 2 Milligrams500milligrams 24hours500µl - - | - - - - - |

| Oxirane, 2-(chloromethyl)-, polymer with α-hydro- ωhydroxypoly[oxy(methyl-1,2- ethanediyl)] | Eyes-Mildirritant Eyes-Mildirritan Skin-Mildirritant Eyes-Mildirritant | Mammal Species Unspecified Rabbit Mammal species unspecified Mammal Species unspecified | - - - - | - 24hours100 Microliters - - | - - - - |
| --- | --- | --- | --- | --- | --- |

Sensitisation

| Product/ingredientname | Routeof exposure | Species | Result |
| --- | --- | --- | --- |
| epoxyresin(MW≤ 700) oxirane,mono[(c12-14- alkyloxy)methyl] derivs Fatty acids, C18-unsatd., dimers,oligomericreaction products with 1-chloro2,3- epoxypropane Oxirane,2-(chloromethyl)-, polymer with α-hydro- ωhydroxypoly[oxy (methyl-1,2-ethanediyl)] | Skin Skin Skin Skin | Mammal-species unspecified Mammal-species unspecified Mammal-species unspecified Mammal-species unspecified | Sensitising Sensitising Sensitising Sensitising |

Mutagenicity

Notavailable.

Carcinogenicity

Notavailable.

Reproductivetoxicity

Notavailable.

Teratogenicity

Notavailable.

Specifictargetorgantoxicity(singleexposure)

Notavailable.

Specifictargetorgantoxicity(repeatedexposure)

Notavailable.

Aspirationhazard

Notavailable.

Informationonlikelyroutesofexposure: Notavailable.

Potentialacutehealtheffects

Eyecontact:Causesseriouseyeirritation.

Inhalation	:Noknownsignificanteffectsorcriticalhazards.

Skincontact:Causesskinirritation.Maycauseanallergicskinreaction.

Ingestion	:Noknownsignificanteffectsorcriticalhazards.

Symptomsrelatedtothephysical,chemicalandtoxicologicalcharacteristics

Eyecontact:Adversesymptomsmayincludethefollowing:painorirritationwateringredness

Inhalation	:Nospecificdata.

Skincontact:Adversesymptomsmayincludethefollowing:irritationredness.

Ingestion	:Nospecificdata.

Delayedandimmediateeffectsaswellaschroniceffectsfromshortandlong-termexposureShort term exposure

Potentialimmediateeffects:Notavailable.

Potentialdelayedeffects:Notavailable.

long-termexposure

Potentialimmediateeffects:Notavailable.

Potentialdelayedeffects:Notavailable.

Potentialchronichealtheffects

Notavailable.

General:Oncesensitized,asevereallergicreactionmayoccurwhensubsequentlyexposedto very low levels.

Carcinogenicity: No known significant effects or critical hazards. Mutagenicity	: No known significant effects or critical hazards. Teratogenicity		: No known significant effects or critical hazards. Developmentaleffects:Noknownsignificanteffectsorcriticalhazards. Fertility effects		: No known significant effects or critical hazards.

NumericalmeasuresoftoxicityAcute toxicity estimates

Notavailable.

# ECOLOGICALINFORMATION

Toxicity

| Product/ingredientname | Result | Species | Exposure |
| --- | --- | --- | --- |
| epoxyresin(MW≤ 700) | AcuteEC501.4mg/l AcuteLC503.1 mg/l ChronicNOEC0.3 mg/l | Daphnia Fish - pimephalespromelas Fish | 48 hours 96 hours 21 days |

Persistence/degradability

| Product/ingredientname | Aquatichalf-life | Photolysis | Biodegradability |
| --- | --- | --- | --- |
| epoxyresin(MW≤700) | - | - | readily |

Bioaccumulativepotential

| Product/ingredientname | LogPow | BCF | Potential |
| --- | --- | --- | --- |
| epoxyresin(MW≤700) oxirane,mono[(c12-14- alkyloxy)methyl]derivs | 2.64to3.78 3.77 | 31 160to263 | Low Low |

Mobilityinsoil

Soil/waterpartition:Notavailable.

coefficient(KOC)

Otheradverseeffects:Noknownsignificanteffectsorcriticalhazards.

# DISPOSALCONSIDERATION

DisposalMethods:Thegenerationofwasteshouldbeavoidedorminimisedwhereverpossible. Disposalofthisproduct,solutionsandanyby-productsshouldatalltimescomplywiththe requirementsofenvironmentalprotectionandwastedisposallegislationandanyregionallocal authorityrequirements.Disposeofsurplusandnonrecyclableproductsviaalicensedwaste disposalcontractor.Wasteshouldnotbedisposedofuntreatedtothesewerunlessfully compliantwiththerequirementsofallauthoritieswithjurisdiction.Wastepackagingshouldbe recycled.Incinerationorlandfillshouldonlybeconsideredwhenrecyclingisnotfeasible.This materialanditscontainermustbedisposedofinasafeway.Careshouldbetakenwhen

handlingemptiedcontainersthathavenotbeencleanedorrinsedout.Emptycontainersor linersmayretainsomeproductresidues.Avoiddispersalof spiltmaterialandrunoff andcontact withsoil,waterways,drainsandsewers.Donotallowtoenterdrainsorwatercourses.Material and/orcontainermustbedisposedofashazardouswaste.

# TRANSPORTINFORMATION

|  | UN | IMDG | IATA |
| --- | --- | --- | --- |
| UNnumber | UN3082 | UN3082 | UN3082 |
| UN proper shippingname | Environmentally hazardous substance, liquid, n.o.s. (epoxy resin (MW ≤ 700)) | Environmentally hazardous substance, liquid, n.o.s. (epoxy resin (MW ≤ 700)). Marine pollutant (epoxy resin(MW≤700)) | Environmentally hazardous substance, liquid, n.o.s. (epoxy resin (MW ≤ 700)) |
| Transport hazard class(es) | 9 | 9 | 9 |
| Packinggroup | III | III | III |
| Environmental hazards | Yes. | Yes. | Yes. |
| Additional information | This productis not regulated as a dangerous good when transported in sizesof≤5Lor≤5 kg, provided the packagings meet the general provisions of 4.1.1.1, 4.1.1.2 and 4.1.1.4to4.1.1.8. | This productis not regulated as a dangerous good when transported in sizes of ≤5 L or ≤5 kg,provided the packagings meet the general provisions of 4.1.1.1, 4.1.1.2and4.1.1.4to 4.1.1.8. Emergency schedulesF-A,S-F | This product is not regulated as a dangerous good when transportedinsizesof≤5Lor≤5 kg, provided the packagings meet the general provisions of 5.0.2.4.1, 5.0.2.6.1.1 and 5.0.2.8. |

Additionalinformation

TransportinaccordancewithADR/RID,IMDG/IMOandICAO/IATAandnationalregulation.

ADR / RID :Tunnel restriction code: (-) Hazardidentificationnumber:90

IMDG:.Thisproductisnotregulatedasadangerousgoodwhentransportedinsizesof≤5Lor

≤5kg,providedthepackagingsmeetthegeneralprovisionsof4.1.1.1, 4.1.1.2and4.1.1.4to 4.1.1.8.

Specialprecautionsforuser:transportwithinuser’spremises:Transportwithinuser’s premises: always transport in closed containers that are upright and secure. Ensure that persons transporting the product know what to do in the event of an accident or spillage.

TransportinbulkaccordingtoAnnexIIofMarpol andtheIBCCode:Notapplicable.

# REGULATORYINFORMATION

Singapore-hazardouschemicalsundergovernmentcontrol

None.

# OTHERINFORMATION

Keytoabbreviations:

  - ATE=AcuteToxicityEstimate

  - BCF=BioconcentrationFactor

  - GHS=GloballyHarmonizedSystemofClassificationandLabellingofChemicals

  - IATA=InternationalAirTransportAssociation

  - IBC=IntermediateBulkContainer

  - IMDG=InternationalMaritimeDangerousGoods

  - LogPow=logarithmoftheoctanol/waterpartitioncoefficient

  - MARPOL = International Convention for the Prevention of Pollution From Ships,1973asmodifiedbytheProtocolof1978.("Marpol"=marinepollution)

  - UN=UnitedNations.

References:Notavailable.

# NOTES

PT.INDOWIJAYASAKTITEGUHhasaresponsibilitytotakereasonablecareforourownhealth and safety and the health and safety of others who may be affected by our acts or omissions. This M.S.D.S. at the date of issue has Health and Safety Information of the product, and how to safely handle and use this product in the workplace.

All information given is our best knowledge, and because we cannot anticipate or control the conditionsoftheenduseofthisproducts,priortousage,eachusermustdeterminebyreviewing this M.S.D.S, Safe Handling and usage of this products in the Workplace.

PT.INDOWIJAYASAKTITEGUHbelievethisinformationtobereliable,andingoodfaith,butno guaranteesorwarrantiesofanykindaremadeastoitsaccuracy,suitabilitytoparticulate applicationsduetovariationsinmethods,conditionsandequipment.WhenPT.INDOWIJAYA SAKTI TEGUHprovides information and service involving skill, assistance, judgment, recommendations, and or advise this is done on the best of our knowledge only; information is not be relied upon.

Full scale testing and performance of the product is the responsibility of the end user. For furtherinformationorclassificationofcertainpointstoensurethattheuserhasmadeaproper assessmentandreasonableprecautionshavebeenapplied,pleasecontactPT.INDOWIJAYA SAKTI TEGUH.